3. Methodology

This section describes our framework for detecting dyslexia across different age groups and eye-tracking devices. Our goal was to create a "hardware-independent" system that works even when switching between different sensors or demographics.

## 3.1 Data Acquisition and Harmonization

We combined three different datasets to test how well our model generalizes:

- Dataset A (ETDD70): Data from 70 children (ages 9–10) including fixation and saccade logs.

- Dataset B (Kronoberg): Raw eye-movement coordinates from 185 second-grade students.

- Dataset C (Adult Cognitive): Raw data from 64 adults to test if the model works on older subjects.

### 3.1.1 The I-VT Protocol (Standardizing Data)

To make the raw data from different devices match, we used a Velocity-Threshold Identification (I-VT) algorithm. This separates eye movements into two types:

- Fixations: When the eye stays steady to process information.

- Saccades: When the eye moves quickly between points.
We used a dynamic threshold (v = mu + 2sigma) to adjust for the different recording speeds of each device.

## 3.2 Feature Engineering

Instead of looking at where someone looks on a specific screen, we looked at how they move their eyes. This makes the model work on any screen size.

- Gaze Stability: Measuring how steady the eye stays during fixations. Dyslexic readers often show more instability.

- Movement Ratios: We calculated the Fixation-to-Saccade Ratio. Because this is a ratio, it stays the same regardless of how long the reading task lasts.

- Reading Regressions: We tracked "backward" eye movements. Frequent re-reading (regressions) is a key indicator of dyslexia.

## 3.3 Machine Learning Models

We tested three different algorithms to find the most reliable one:

- Random Forest: Used as a starting point to see which features were most important.

- Support Vector Machines (SVM): A linear model used to find broad patterns and avoid focusing on "noise" from the hardware.

- XGBoost (Final Choice): This was our best-performing model. It uses advanced error-correction to ignore hardware differences and focus only on the biological signals of dyslexia.

## 3.4 Experimental Validation

We tested our system in three steps:

- Step 1: Internal Testing. We trained and tested the model on the same group to set a baseline accuracy.

- Step 2: Zero-Shot Transfer. This was the "stress test." We trained the model on children (Dataset A) and tested it on a completely different group (Dataset B) without any extra training. We used Quantile Normalization to make the data from different devices look statistically similar.

- Step 3: Visual Analysis (PCA). We used Principal Component Analysis to map the reading patterns of both adults and children. The results showed that dyslexic reading "signatures" look the same across all ages, proving a single model can work for everyone.

## 3.5 Summary of Contributions

Our methodology provides a way to screen for dyslexia that does not depend on expensive or specific hardware. By focusing on movement ratios rather than screen coordinates, we have created a tool that is ready for real-world, multi-age use.